CONTENT

1	Scope of Application	3

2	Purpose	3

3	Definitions	3

3.1	Plant equipment erection:	3

3.2	Crane lifting study:	3

4	Applicable documents, notes	3

5	Scope and Responsibilities	4

6	Planning of Lifting and Erection Activities	4

7	Design and arrangement of the shims:	4

8	Anchor plates/shear cross on fixed and sliding supports	5

9	Hook on for transport and lifting	5

10	Erection of rotating plant equipment	5

11	Cold box erection	5

11.1	Erection of prefabricated Cold boxes (package units)	5

11.2	Erection instructions for plant equipment inside of the cold-box (site erection)	5

12	Surveying	6

13	Permissible tolerances for alignment	6

13.1	Tolerances for vertical plant equipment	6

13.2	Tolerances for horizontal plant equipment and rotating equipment	7

14	Inspection	7

14.1	Pre-inspection activities (not limited to):	7

14.2	Intermediate inspection activities (not limited to):	7

14.3	Final inspection activities (not limited to):	8

15	Equipment Preservation	8

16	Quality records	8

# Scope of Application

This specification applies to the erection, alignment and surveying of plant equipment at Construction Site/Modular Construction Yard.

# Purpose

This specification establishes the minimum requirements for proper erection of plant equipment.

# Definitions

CLIENT			The company who awarded the contract to COMPANY.

COMPANY			Linde HQ

CONTRACTOR 		means engaged by Company for execution of heavy lifting work

SUB-CONTRACTOR		means engaged by Contractor, performing parts of heavy lifting work

## Plant equipment erection:

The term 'Plant equipment erection' includes the erection of e.g. cold boxes, pipe skids, heat exchangers, air coolers, vessels, columns, rotating equipment including their drives on foundations or supporting structures, as well as their alignment.

For the erection of rotating equipment additional job specific instructions ("installation manuals") from the equipment manufacturer have to be considered. It is CONTRACTOR'S responsibility to make sure that those "installation manuals" are handed over to him before detailed planning of the work starts.

Final alignment of pumps, compressors and their drives is excluded from this specification.

## Crane lifting study:

The crane lifting study is a basic requirement for planning and safe execution of erection and dismantling processes. The study shall be prepared by CONTRACTOR for erection of equipment >50 tonnes, for lifting activities over life plant equipment and the use of more than one crane. The study need to take all safety aspects, local conditions, access, transport routes and the erection sequence into consideration. Independent from the equipment weight, CONTRACTOR has to perform a rigging check prior carrying out any lifting activities.

# Applicable documents, notes

The following documents / notes shall be considered:

- All specifications, test documents and documentation as listed in the list of attachments

- All written agreements made with the CONTRACTOR (authorised personnel) during the progress of erection

- All applicable work and erection instructions

- Acceptance sheets provided by COMPANY

- &AZ-W-SC 4501 (EN) 	Construction Specification – Cold Box Installation

- &CG-W-GE 9222 (EN)  List of attachments ITP Equipment

# Scope and Responsibilities

The Scope and Responsibilities are defined in the specification &AA-W-SY 2201 (EN) "Scope of work"

# Planning of Lifting and Erection Activities

All construction equipment, tools and lifting devices must be certified, in high quality and shall be inspected and serviced regularly. Applicable rules, laws and regulations shall be considered. Respective evidence such as crane inspection books, static and inspection reports have to be presented. Only qualified and approved lifting personnel is allowed to execute the work.

The lifting and erection shall follow a detailed plan (to be submitted to COMPANY for review) and shall consider at least (but not limited to) the following aspects:

- consider site conditions (suitable for transport and lifting / plant in operation)

- slings shall slide without any hindrances

- use of tailing cranes and / or suitable tailing equipment for lifting of vertical plant equipment from horizontal position (e.g. columns, vessels, etc.)

- use guiding ropes / cables, spreader beams

- consider the "Orientation of nozzles"

- alignment according the tolerances as shown in para. 13 or as per manufacturers requirements (the more stringent requirements shall apply)

- remove temporary supports and loose parts from the equipment before lifting

- anchor bolts shall be aligned in holes of the equipment based on nozzle orientation requirements

- nuts on sliding support must be loose, a 2 mm gap shall remain between the washer and the first nut, and the nuts must be secured with a locknut

- the contact surfaces of the fixed and sliding supports must be free of foreign materials

- it is non-permissible to perform any kind of hot and cold work to anchor bolts

- modification of bolt holes on the plant equipment requires written approval from COMPANY

- anchor bolts must be lubricated, tightened with specified torques (see Manufacturer drawings) and documented in the final acceptance sheet

- installation of sliding plates.

The lifting and erection shall follow a sequence, implemented in the CONTRACTORS lifting study.

Note:	A column being erected undressed and without installed internals could be affected by wind (twisting). For such cases, precautions are defined in the Equipment Specification and need strictly be followed.

# Design and arrangement of the shims:

Unless otherwise specified, the shims shall be carbon steel material. In some cases (only if specified by COMPANY), the shims can be from aluminium or stainless steel.

The shims must be exempt from burrs, levelled and must rest on the total surface.

Maximum 5 shims shall be placed on top of each other. The minimum size shall be 50 x 100 mm.

The space between the top edge of the foundation and the base plate, base frame or base ring of the equipment must be at least 30 mm and maximum 50 mm.

The shims shall be placed at both sides of the anchor bolts to make sure that the maximum possible force is transferred between the equipment and the foundation and that the grouting work is not impaired.

Shim plates must be installed with a set-back of 10 mm to the equipment base plate.

The first shims may be set and aligned in cement mortar or non-shrinking grout. The grout must be completely cured before the plant equipment is placed on top.

# Anchor plates/shear cross on fixed and sliding supports

Anchor plates/shear cross on fixed and sliding supports must be fitted together, aligned and checked (X, Y, Z coordinates). Cavities shall be filled with concrete and the concrete shall be cured before erection. The guides for the sliding supports must be welded after erection (unless otherwise specified), ensuring clearance (min 2mm / max. 4mm) on both sides.

Foundations and equipment parts coming into contact with grout material shall be free from oil and grease.

# Hook on for transport and lifting

Transport and lifting shall only be allowed by comply with the manufacturer's information for hook on points as shown in the equipment, transport and / or lifting drawings.

Only certified lifting equipment shall be used. Evidence for the certification shall be in place.

Stainless steel equipment must be protected from carbon steel contamination!

Important! Before lifting of thin, long and slim plant equipment, preventive measures as specified by COMPANY or manufacturer shall be taken to avoid buckling (e.g. reinforcements, use of wide plastic lifting belts).

All kind of damage to the surface of plant equipment shall be avoided! In case of damages caused by the CONTRACTOR, these have to be rectified by CONTRACTOR without additional cost for COMPANY.

# Erection of rotating plant equipment

Instructions given by the manufacturer of the rotating equipment shall be taken as essential. Deviations may cause a defect to the rotating equipment.

In connection with concrete foundations, the bottom shim plate shall be set and aligned in cement mortar or non-shrink grout.

Before erection the following points shall be checked and verified:

- Accessibility of drain plugs and bottom covers

- Base frames must be checked for grouting- and vent openings

- Protective covers fitted to flanges must be checked for damages. Protective covers shall only be removed after instruction by COMPANY

- Pumps with connection to vacuum insulated piping, supplied prefabricated, shall only be erected shortly prior to installation of this pipe work and shall only be doweled, respectively anchor bolts grouted after stress-free connection of this pipe work so that corrections are still possible.

# Cold box erection

## Erection of prefabricated Cold boxes (package units)

Construction Specification – Cold Box Installation (&AZ W- SC 4501) shall be considered for the installation of Cold Boxes. This specification details the special requirements and works to be completed before lifting of Cold Boxes.

Cold boxes shall be lifted with a lifting and tailing crane using double crane hook. Additional loads on lifting rope due to the distance between lifting and tailing cranes shall be considered in the rope design.

## Erection instructions for plant equipment inside of the cold-box (site erection)

The supporting structures for vessels, heat exchangers and columns must be located, aligned, stuffed with mineral wool (in case of hollow profiles) and anchored to the bottom of the cold-box in accordance with the layout drawing.

The position of the bolt holes on the supporting ring or supporting skirt shall be checked against the "Orientation of nozzles"' and the base-ring holes on the vessel.

The evenness and the alignment of the supporting ring on which the vessel base ring rests need to be guaranteed.

Vessels, heat exchangers and columns shall be placed on the supporting structures, temporarily supported, aligned and fastened with the specified bolts which shall be secured with locknuts.

Attention: For aligning columns refer to &AZ-W-SC 2202 (EN) " Construction Specification – Checking and aligning of Rectification Columns in Cold Boxes".

For columns guided at the metal casing, the bracing shall be installed at the appropriate height according to the project-specific documentation.

At vessels / heat exchangers which are mounted on columns or other vessels, correct installation of the fixed and sliding supports shall be ensured; bolts shall be secured with locknuts. No welding is permitted on the shell of pressure vessels without written approval by COMPANY. After erection of vessels / heat exchangers on columns, these shall be surveyed again and, if necessary, realigned.

For plant equipment which is suspended from the roof structure or a beam, the exact height shall be levelled, the guiding supports have to be fitted after alignment and the bolts shall be secured with locknuts.

# Surveying

Surveying must be carried out by qualified personnel and approved tools (e.g. levelling device, theodolite, lasers). Qualification certificates shall be available.

Surveying shall be done without direct sun radiation, i.e. preferably in the morning.

Surveying of columns and vessels concerning verticality or horizontality shall be done according to the following instructions. CONTRACTOR is free to choose between these three methods. Any deviation of execution requires written approval from COMPANY.

- Measuring of tangent lines of the outer surface.

At minimum 2 vertical measurement lines (tangent lines) - 90° displaced are required, one at the base ring and one on top of the column. This method is not allowed to be used if the equipment is delivered with applied passive fire protection at the surveyed locations.

- Measuring of nozzle flanges.

The main nozzle flanges must be always surveyed, like inlet nozzle and outlet nozzle. If existing, additional nozzle flanges shall be chosen according to height location, ideally one close to the ground and the other one close to the top. If possible, additional nozzle flanges 90° displaced shall be surveyed. Nozzle flanges for instrumentation connections don’t need to get a survey, except it is requested by COMPANY’s personnel.

- Measuring of centre lines.

CONTRACTOR shall mark the centre line directly on the equipment (with duct tape or similar). It is sufficient that the bottom and top of the equipment is marked. Same procedure 90° displaced. Surveying of both centre lines. CONTRACTOR is responsible to remove or clean the marking after successful surveying and signed protocol by COMPANY.

# Permissible tolerances for alignment

## Tolerances for vertical plant equipment

For columns inside of cold boxes refer to &AA-W-SC 2202 (EN) "Specification for checking and aligning of rectification columns in cold boxes".

Height deviations: +/- 1 mm, measured on the bottom edges of the base ring, supporting brackets, base plates or base frames.

Vertical deviations: 0.3 mm/m for cold-boxes and for vertical plant equipment.

X, Y co-ordinate deviations: +/-3 mm in the "X" and / or "Y" directions, measured at the reference
co-ordinates marked on the foundation and recorded.

Columns and vessels shall be measured at minimum 2 vertical measurement lines (tangent lines) - 90° displaced, at least at the base ring and on top of the column.

## Tolerances for horizontal plant equipment and rotating equipment

Height deviations: +/- 1 mm, measured at 2 points each on the bottom edges of the fixed support, sliding support, base plate or base frame.

Horizontal deviations: +/-0.3 mm/m, max. +/-1 mm measured over the complete length.

X, Y co-ordinate deviations: +/-3 mm in the "X" and / or "Y" directions, measured at the reference
co-ordinates marked on the foundation and recorded.

Instructions given by the rotating equipment manufacturer shall be taken as essential. Deviations may cause a defect on the equipment.

Deviations from the specified tolerances shall be immediately reported to the COMPANY!

# Inspection

CONTRACTOR is obliged to perform pre-, intermediate- and final inspections to ensure the quality of the work. CONTRACTOR shall show all activities in the Inspection & Test Plan (I&TP).

## Pre-inspection activities (not limited to):

- Lifting study (if required) available and approved.

- Crane driver (operator) has valid licence.

- Completeness of the erection material. Missing parts shall be immediately reported to COMPANY!

- Weight and dimensional check of the plant equipment to be installed. Internal and/or external parts fitted to plant equipment at site must be considered (if applicable).

- All tools and equipment like ropes, slings, shackles, and spreader beams shall be fit for purpose, static proof (certificates) shall be available

- Transport routes, loading and unloading points, soil and weather conditions, clearance heights and widths shall be checked. All obstructions in the transport route and erection area shall be removed.

- Check that all participants (involved in the lifting process) read and understand the lifting study and Job Safety Analysis.

- Foundations and support check.

- Check all kind of lifting equipment used for job execution including the crane book.

- Check equipment to be lifted and confirm the consistence to the manufacturer's approved drawings and or transport sketches.

- Check the weather forecast for the erection period.

- Check the equipment for damage and confirm that no loose parts are placed on the equipment.

- Measure and record the erection height after placement of the shims.

## Intermediate inspection activities (not limited to):

- Check the load security

- Re-confirm clearance from control room (applicable for plants in operation)

- Check the safety precautions for the actual lifting process

- Check position of fixed / sliding supports for horizontal plant equipment

- Check the placement of the shims (position, size, quantity, material, etc.)

- Check the correct orientation of the plant equipment and perform constant surveying.

## Final inspection activities (not limited to):

- final alignment check

- Check and record the tightening torques of the anchor bolts

- Measure settling of foundation (especially for heavy plant equipment such as tanks, columns, etc.)

- Check fixed, sliding and loose supports

- Prepare acceptance protocol and transfer to COMPANY.

# Equipment Preservation

The preservation of equipment must be carried out in accordance with  all project related preservation specifications, procedures and manufacturers instructions.

# Quality records

The CONTRACTOR is obliged to record the "ACTUAL" values of X-, Y-, Z- co-ordinates and the related deviations of the installed and aligned equipment. Therefor COMPANY will provide acceptance sheets. The certificate for Construction Completion shall be used and signed by CONTRACTOR and for final acceptance by COMPANY.

All "originals" of the acceptance sheets, approvals and quality records must be handed over to COMPANY.