.

Hook-Up Piping

Contents

1	Scope	3

2	Purpose	3

3	Definitions	3

4	Referenced Documents	4

5	Hook-up Philosophy	4

6	Golden Welds	4

7	Hook-up Requirements	4

7.1	Insulation	4

7.2	Examples for hook-up types	5

7.3	Hook-Up Tolerances	6

8	Pre-fabrication of Hook-up spools	6

8.1	Sandblasting and Coating	6

8.2	Hook-up spool marking	6

9	Pressure Testing	6

10	Tie In's	6

11	Storage and preparation for transport	7

12	Documentation and Records	7

13	Distribution	7

# Scope

This specification is applicable for hook-up of piping on construction sites and shall be used in conjunction with &AA-W-SC 2401 (EN) "Construction Specification for Metallic Piping".

# Purpose

This Specification defines minimum requirements as well as responsibilities and workflow prior and during the performance of hook-up activities on sites in order to ensure that hook-ups are carefully planned and professionally performed.

Welded hook-ups may be required for piping connections

- between pre-assembled units and site run piping or

- site run piping tied-in to existing plant piping or equipment.

This specification is applicable to all piping classes with following exclusions:

- instrument piping downstream of the first piping block valve

- flexible hoses

- sanitary piping

- plastic piping (GRP, GRE etc.)

# Definitions

| COMPANY | Linde LE or Linde LE Entity |
| --- | --- |
| CONTRACTOR | The party which is awarded a contract from LE |
| Construction Site | Main term for fabrication sites for prefabrication or field assembly as well as existing plants where repair work or reconstruction is carried out. |
| Hook-Up | Welded connection of pre-fabricated plant sections (modules) at construction site either directly (module to module) or by installation of interconnecting piping spools. |
| Tie In | Welded connection of new piping to existing plant piping or equipment |
| FW | Field Weld = assembly weld at construction site. |
| FFW | Field Fit Weld = assembly weld where pipe ends are provided with fit lengths. |
| Golden Weld | Weld with 100% NDE and additional precautions in lieu of pressure test. |
| GTAW | Gas Tungsten Arc Welding |
| DPT | Dye Penetrant Testing |
| MPT | Magnetic Particle Testing |
| NDT | Non Destructive Testing |
| PWHT | Post Weld Heat Treatment |
| RT | Radiographic Testing |
| UT | Ultrasonic Testing |
| WPS/PQR | Welding Procedure Specification / Procedure Qualification Record |

# Referenced Documents

&AA-W-SC 2401 (EN) "Construction Specification for Metallic Piping"

# Hook-up Philosophy

Pre-defined plant sections may be completely pre-fabricated, pressure tested, re-instated and brought to “Mechanical Completion” status at the pre-fabrication site. Hook-up points are located at the battery limits of these pre-fabricated plant sections.

Welded Hook-up's may also be required for tying in new piping to existing plant piping or equipment. In this case different code requirements may become applicable. All details regarding preparation, welding and testing will have to be agreed upon in advance with CLIENT and concerned authorities.

Hook-up's shall be performed at construction site and shall be welded joints in general. The hook-up spools shall be prefabricated, flushed, pressure tested if required and coated in the pre-fabrication shop.

# Golden Welds

The amount of Golden Welds in a project shall be kept to a minimum.

In those cases where both, hydrostatic and pneumatic testing, appears to be impracticable or impossible, the CONTRACTOR may propose Golden Welds in lieu of pressure testing. This requires COMPANY-QA/QC approval on a case-by-case basis. Golden Welds shall be requested by CONTRACTOR via a Site Query and registered in the Golden Weld Register.

Following conditions as a minimum have to be met for Golden Welds:

a)	COMPANY approval available on site query

b)	Fit-up visual inspection, in-process visual inspection as well as final visual inspection by CONTRACTOR-QC (Report required)

c)	Root welding and hot pass shall be GTAW process

d)	100% volumetric NDT (RT or UT, depending on wall thickness)

e)	100% surface inspection by DPT or MPT

f)	Detailed documentation of Golden Welds incl. all NDT and test reports as a part of the As 	Built Documentation

g)	Update of a Golden Weld Register entering line-no. and relevant weld no

# Hook-up Requirements

Pipe ends for FW shall be bevelled in the pre-fabrication shop according to the Welding Procedure Specification (WPS).

Hook-ups shall be assembled stress free and no excessive mechanical force shall be used to obtain alignment.

Hook-up welds in stainless steel, duplex steel and >5%Cr-Alloy lines will have to be purged during welding in order to prevent oxidation of the root. Therefore the placement and removal of the purging equipment inside the lines requires prospective planning by the CONTRACTOR responsible for hook-up activities.

Prior to assembly the hook-up spools as well as the hooked up pipe ends shall be checked for cleanliness and shall be confirmed in writing on appropriate reports by CONTRACTOR and verified by COMPANY.

## Insulation

Insulation shall be set back from weld bevel by 500 mm each side for DN ≤ 200 (8"), and 700 mm each side for DN > 200 (8").

For welds with PWHT, insulation shall be set back from weld bevel by 700 mm each side for DN ≤ 200 (8"), and 1000 mm each side for DN > 200 (8").

All open ends of insulation shall be sealed professionally to prevent ingress of water. Those closures shall be removed prior the final insulation of the hook-up section.

500 (700)

Insulation

weld bevel

(1000)

## Examples for hook-up types

Unit 1

Hook-up weld (FW)

Unit 2

FW

Hook-up spool

FFW

FW

FW

Hook-up spool

FFW

FFW

Hook-up spool

FFW

## Hook-Up Tolerances

radial (z)

radial (y)

axial (x)

axial (x) :		+ 0 /-3 mm

radial (y, z) :		± 2 mm

Tolerances in reference to plant co-ordinates (Electronic measuring Equipment shall be used)

Fit-up length for FFW is 100 mm for one end; other end shall be bevelled in pre-fabrication shop.

Open pipe ends shall be closed with plastic caps, sealed with tape.

# Pre-fabrication of Hook-up spools

Hook-up spools shall be prefabricated in pre-fabrication shop according to isometric drawings. FW and FFW shall be defined in the isometric drawing by CONTRACTOR.

For FFW, the cut end, which do not have the fit-up length, shall be bevelled at pre-fabrication shop. The second end shall have approximate 100 mm fit-up length.

Spool sections shall be marked according to Paragraph 8.2 below.

## Sandblasting and Coating

Shop prefabricated hook-up spools may be coated prior to shipment if:

- Spools have been pressure tested at pre-fabrication site

- Seamless piping components are installed in spools and welds remain only prime coated (one layer).

50 mm at each side of hook-up weld bevel (measured at the bevel end), shall remain uncoated (fit-up length to be considered). For hook-up welds to be heat treated 300mm each side of the weld bevel, however a minimum distance of 150mm from the heating elements shall remain uncoated. Hook-up spools subject to PWHT shall be shot blasted and prime coated only after PWHT completion.

## Hook-up spool marking

Each hook-up spool shall be designated to a hook-up number.

Hook-up numbers shall be indicated on the shop isometrics and durably marked on the spools.

The marking of hook-up spools shall contain:

- The construction area no.

- Hook-up no.

# Pressure Testing

Pressure testing of hook-up spools resp. hook-up welds is required wherever possible/practical.

For certain utility lines a service test with Air, N2 or process fluids may be acceptable upon COMPANY's approval.

In those cases where the hook up spool will be subject to pressure test at construction site, a pressure test at the pre-fabrication shop is not required.

# Tie In's

In case of tying in new piping into existing plants, all details regarding test pressure, test circuit, termination points and Golden Welds etc. shall be agreed upon with CLIENT and if required by the concerned authorities.

Thorough preparation and examinations of the hook-up points in existing plant piping will be required. This may include but not be limited to wall thickness checks, surface crack inspections at weld bevels, metallographic examinations etc., and the release for execution or continuation of tie-in work shall be agreed by all parties concerned.

# Storage and preparation for transport

To ensure the correct supply of hook-up spools from the pre-fabrication shop to their final destination at construction site the following requirements shall apply as a minimum:

- All pre-fabricated hook-up spools shall be listed by the CONTRACTOR with the following information:
-  Hook-up no.
-  Construction Area no.
-  Delivery/Inspection Status

- Suitable storage conditions shall be maintained in order to prevent damage in particular of coating and insulation and ingress of debris/moisture.

- Open pipe ends shall be closed with plastic caps, sealed with tape.

# Documentation and Records

Hook-up spool register with backup documentation:

- Isometric drawings

- NDT / PWHT reports (including PMI, Hardness test etc. if applicable)

- Pressure testing reports

- WPS/PQR, welder list