Please use the MS Word tool bar  "Data Input Dialog" under register "File 'Linde LE'" to fill in this document.

Internal Installation and Modification

CONTENT

1	Scope of Application	3

2	Purpose	3

3	Definitions	3

3.1	General	3

3.2	Equipment modifications	3

3.3	Installation of Internals	3

4	Applicable documents, notes	3

5	Procedure	3

5.1	Preparation general	4

5.2	Preparations for Installation of Trays	5

5.3	Transport / lifting	5

5.4	Temporary storage	5

5.5	Installation	6

5.6	Flange connection	6

6	Inspections	7

6.1	Pre-inspection	7

6.2	Intermediate inspections	7

6.3	Final inspections	7

7	Tolerances	8

7.1	Exceeding the tolerances	8

8	Quality records,	8

# Scope of Application

This specification applies to the installation, modification and handling of internals for vessels as trays and other parts inside of plant equipment at construction site.

# Purpose

This specification establishes the minimum requirements for proper installation of internals and for modifications.

# Definitions

## General

For particulars regarding CLIENT, COMPANY, CONTRACTOR, etc. refer to EXHIBIT A "SCOPE OF WORK".

Shall = The term "Shall" has the same meaning as "have to " (According to ASME)

## Equipment modifications

Equipment modifications cover the complete installation of prefabricated equipment parts including NDT, pressure testing, executed according to the related technical documentation and specifications as listed in 'List of Attachments'.

## Installation of Internals

The term "Installation of Internals" includes the installation of internals such as trays, demisters, packing (e.g. MELAPACK) for new and existing equipment.

# Applicable documents, notes

    - All specifications, test documents and documentation as listed in the "list of attachments".

    - All written agreements made with the CONTRACTOR (authorised personnel) during the progress of installation.

    - Work and installation instructions (e.g. from manufacturer).

    - Work instructions for modifications

    - LS 103.10.T03	General Tolerances

    - &AZ-W-QA 2204 (EN)	Stationary Equipment Box-Up

    - &AZ-W-QA 2801 ((EN)	Acceptance Sheet Internals

    - &AZ-W-SC 2401 (EN)	Construction Specification for Metallic Piping Installation

    - &AZ-W-TT 9403 (EN)	Construction Schedule

# Procedure

The implementation of plant equipment installation takes place according to the Construction Schedule after release of the corresponding plant equipment and, where applicable, according to instructions of COMPANY.

Releases and priorities during installation shall be coordinated between CONTRACTOR and COMPANY.

When working within plants or parts of plants in operation or within their vicinity and with the simultaneous working of a number of CONTRACTORs at the same site, co-ordination with the responsible coordinator and safety specialist is necessary before starting work.

## Preparation general

- By taking in charge of the internals, trays and installation materials CONTRACTOR accept the responsibility for proper temporary storage, transport and installation.

- Unprotected storage is prohibited and all parts shall be protected against weather and other adverse influences.

- All parts for the installation shall be stored in an area adjacent to the particular plant equipment to facilitate an optimal assembling and installation sequence and to avoid damage to those parts.

- Provision of appropriate material for temporary supports of the internals (trays, etc.), if equipment can be stored near the place of installation (e.g. square-shaped timber, sleepers etc.)

- Unpacking and checking following items at least 4 weeks before start of installation for:

- damages of the delivered equipment

- the completeness of the installation material such as demister, trays, bolts, fastening screws, washers, nuts, etc. and any special tools required.

- the planeness of the delivered supporting rings and clamps

Missing or damaged parts and deviations shall be immediately reported to COMPANY!

- Check weight and dimensions of tray segments and internals based on the packing lists, weighing cards or shipping documentations, or weigh them on site. The weight quoted in the shipping documentation must be compared with the weight in the drawing (calculated drawing weight +/- 10 %). If larger deviations are found, COMPANY shall be contacted for clarification. The weights stated in tendering documentations shall not be used.

- The load capacity and sufficient free space on the transport routes and crane standing areas, lifts and gratings shall be checked by CONTRACTOR.

- Check the availability of suitable material for temporary supporting of the internals during intermediate storage at the installation site in the working range of the lifting crane or lift or at the intermediate storage area (timbers, sleepers, protection covers, etc.).

- Check the blind list for each vessel/column provided by OWNER. Deviations shall be reported immediately to COMPANY.

- Check the availability of cranes, hoists, winches, jacking and lifting equipment, tracking devices, transport equipment, installation aids and installation material, such as lifting beams, shackles, cables, chains, hooks, grabs, sleepers, excavator mats, shims, scaffolding, working platforms, measuring instruments, walky-talkies, etc. (provision of lifts/elevators outside of columns for material lifting only with a repair service contract within 12 hours)

- If CONTRACTOR's equipment is not available promptly, the costs for waiting time shall be borne by the CONTRACTOR.

- Check the weather forecast for the installation period.

- Provision of appropriate weather protection, if necessary, to avoid damages on internals and moisture inside equipment.

- Establish safety zones by setting up barriers

- In case that two or more teams are working inside the column at the same time, the team, working below must be protected against falling parts (e.g. scaffolds inside equipment).

- Inspection of the local condition and application of scaffolds, platforms and lifts in an early stage.

- Lifting devices which are installed inside columns/vessels for rescue purposes have to be approved for transportation of persons and accepted by notified body and shall match to the fire brigade's equipment.

- Stainless steel equipment must be protected against Carbon steel contamination!

## Preparations for Installation of Trays

- Prior to installation of the trays the interior of the column shall be clean and free from debris, welding residuals etc.

- All supporting rings and clamps must show a clean and plane surface. The allowable dimensional tolerances for tray installation stated in the manufacturer's documents shall apply. If no tolerances are stated, the Linde Standard LS 103-10.T03 (EN) "General tolerances" applies.

- If required, the trays shall be cleaned before installation. Depending on type of tray, cleaning may not be necessary.

- Before installation and assembling of the trays all part numbers of the tray parts shall be carefully checked acc. to the related drawings for installation and assembling.

- Tray installation and assembly shall be performed by qualified and skilled personnel with sufficient experience.

- CONTRACTOR is responsible for all damages occurring during these activities.

- If requested by COMPANY or in technical documentation, preservation protection shall be removed from trays.

## Transport / lifting

- All construction equipment, tools and lifting devices must be in proper condition, certified and have to be inspected and serviced regularly. Respective evidence such as crane inspection books, static and inspection reports have to be presented. Only qualified and approved lifting personnel is allowed to execute the work.

- Transport and lifting shall only be allowed by using the manufacturer's information for hook on points (e.g. lifting brackets or lifting lugs) as shown in the transport and / or lifting drawings.

- Only certified lifting equipment shall be used. Evidence for the certification shall be available.

- Important!
Before lifting of thin, long and slim plant parts, preventive measures shall be taken to prevent buckling (e.g. reinforcements, use of wide plastic lifting belts).

- Damage of the transported / lifted items shall be avoided!

## Temporary storage

In case of temporary storage, following requirements shall be considered:

- sufficient space

- allocation in installation areas

- sufficient accessibility

- Separate storage of stainless and carbon steel

- weather protection, if required

## Installation

- Check, that all isolating blinds according blind list are installed.

- Risk of suffocation! Vessels/columns can be filled with nitrogen. Measurement of oxygen concentration before entering vessels/columns and continuous measurement during the installation phase with a direct indication device.

- Precondition for the release of installation are the finished preparation (See 5.1 and 5.2) and pre-inspection (See 6.1).

- Swinging levers (davits) can only be used after acceptance of CLIENT. The maximum allowable loads shall not be exceeded.

- Inside the column only safety-low voltage transformers shall be in service.

- Installation shall proceed from the bottom to the top of the column.

- For the installation of internals (trays, demister etc.) manufacturer's specifications, procedures and guidelines are applicable.

- Apply sealing strips as specified by manufacturer. After installation of the down comer wall or walls (in case of multi-pass trays) the sealing strips are to be applied on top of the supporting ring. Regarding the down comer wall or walls, the sealing strip shall be slipped over the bolts. Metal to metal sealing is sufficient unless gaskets or sealing strips are demanded by the relevant specification.

- Prior to placing the tray segment onto the supporting ring, the clamps shall be pre-assembled at the tray segments.

- The different tray segments shall be bolted together inside the column, and then centered on the supporting ring. Care shall be taken for the alignment of the down comer wall. When the tray is correctly placed, it shall be bolted down with the clamp to the supporting ring. All slots, holes or gaps shall be stuffed with sealing strips and covered with the covering plates and for being sealed liquid-tight.

- All nuts shall be uniformly tightened and secured. Manufacturer’s specification shall be observed.

- Man-ways in down comers shall be properly installed, sealed and be removable from the top.

- Perforated trays shall be plane and undistorted shall be free of burrs. The admissible tolerances shall not be exceeded.

- Special care shall be taken for the correct installation and assembling of liquid distribution nozzles or trays and draw-off and collection trays.

- All tray installation joints shall be tight.

- For two-pass and four-pass trays, the tray segments beneath the down comer walls shall be installed first and then bolted to the down comer wall by means of stiffening plates.

- On perforated trays which are delivered with down comer seal areas, the down comer seal areas be be bolted to the tray segments before the down comer wall and/or down comer walls. After that, the clamps at the supporting ring shall be bolted on.

- Man-ways of trays shall be left open until COMPANY releases for closing them.

- Remove isolating blinds and re-connect flanges (incl. documentation) after complete installation, inspections and release by COMPANY.

## Flange connection

Refer to &AZ-W-SC 2401 (EN).

# Inspections

## Pre-inspection

Pre-inspections take place before starting the installation phase of internals and include following activities (not limited to):

- Job safety (construction site, equipment):

- Inspection, whether appropriate safety-related equipment is available (direct indication device for oxygen concentration, harness, walkie-talkies etc.)

- Inspection, whether appropriate equipment for personal rescue is available

- Inspection of the local conditions (barriers, etc.)

- Measurement of oxygen concentration before entering column/vessel

- Inspection of damages at internals and inside/outside column/vessel

- Inspection that maximum load of platforms/ gratings are not exceeded

- Qualifications of personal (resume)

- Timely Application for all necessary work permits (hot work, installation and entry of column).

## Intermediate inspections

These activities are (but not limited to):

- Continuous supervision of quality according standards and specifications

- Continuous supervision of installation work

- Load securing for transports

- Continuous supervision of safety measures

- Continuous measurement of oxygen concentration

- Inspection of releases for internals

- Visual inspection of the internals for damage; produce a damage report if necessary

## Final inspections

These activities are (but not limited to):

- List and schedule any remaining work.

- Dismantle the temporary installation arrangements

- Prepare acceptance protocol (signed by COMPANY) and transfer to COMPANY.

- Release of the follow-on work by COMPANY.

- After installation and assembling, each tray shall be checked and inspected together with COMPANY.

- The actual measurements and dimensions are to be recorded in the acceptance protocol. See &AZ-W-QA 2801 (EN).

- Box up and preparation of an acceptance report according &AZ-W-QA 2204 (EN).

# Tolerances

For tolerances the relevant manufacturer drawings and specifications are valid for the installation and assembling work for trays. If specifications do not exist, LS 103-10.T03 (EN) "General tolerances" of COMPANY shall be used.

## Exceeding the tolerances

Deviations from the specified tolerances require written approval from COMPANY.

# Quality records,

The CONTRACTOR is obliged to record the "ACTUAL" dimensions of the installed parts. The acceptance reports &AZ-W-QA 2801 (EN).shall be used and signed by CONTRACTOR and for final acceptance by COMPANY.

All "originals" of the acceptance reports, approvals and quality records must be handed over to COMPANY.